Hi George,

Here are my calculations for the costs.  I will also give you the excel spreadsheet that contains all of this data, but this document will be a better record of where everything comes from and what assumptions I have made.  I have broken the cost analysis down into 4 parts: sample preparation costs, cost of polony amplification, cost of sequencing reaction, and number of polonies per slide (area of 20 x 60 mm).

- Sample Preparation.   These are the costs for doing  random priming reaction with genomic DNA or sorted chromosomes.  These cost do not scale with the number of bases sequenced per patient, only with the number of samples prepared.  The costs listed below are all “off the shelf” costs.  I also calculated labor costs, assuming a cost of $80,000 per year for each employee.  The number is on the high side to account for salary, benefits and other overhead associated with having an employee.

- Cost of polony amplification.  I am calculating the cost of amplifying a 20mm x 60mm x 2 micron gel attached to a glass microscope slide.  For bigger or smaller areas, the cost should scale with the area.  Each reagent is listed below with information about its vendor and catalogue number.   The costs are all off the shelf costs.  I have also included what I estimate the cost of using a PCR cycler.  I assume an 80% duty cycle, and amortize the machine over 5 years.  This cost is negligible.

- Cost of Sequencing Reaction.  Here I assume the volume of sequencing reagents is roughly 10 times the volume of the polony gel.  This is my best estimate based on the concentration of fluorescent nucleotide that I like to use, the diffusion rate of the nucleotides and the possible number of bases that could be incorporated in one addition.  Furthermore, earlier we had estimated that 4 base additions would give us approximately 2.5 bases of information, so I am assuming 500 single base extension.  I am also calculating the cost of scanning.  For this calculation, I am assuming that we are using a modified Axon scanner.  I had talked to an Axon engineer when they sold us the machine and he said that they could get 1 micron resolution with a few easy changes.  I described our needs, and he estimated that the scan would only take about 5 times longer than a 5 micron resolution scan.  I assume we use both channels so that 2 scans takes 25 minutes or 1 scan takes 12.5 minutes.

- Number of Bases Read Per Slide.  Earlier versions of this assumed 38 million polonies per slide.  This version assumes 18 million, as this is the number of informative polonies that we have actually achieved.

- Final Cost Calculations: